	[image: image1.png]




	[image: image2.png]/A GIKEN

SIEENT PIER Up-catod ey 2000





	[image: image3.png]




	 
	Clear Piler CL70 (Type 4) Specification
	 

	[image: image4.png]




	 
	The CL70 is specially designed for the "Overhead Clearance Method".
	 

	
	[image: image5.jpg]




	

	[image: image6.png]





	 
	[image: image7.jpg]270
230

1210
1521





	 
	Silent Piler
 

[image: image8.png]



Sheet Pile Range

U Profile 400 mm

[image: image9.png]



Max. Installation Force

 

700 kN 

Max. Extraction Force

 

750kN

[image: image10.png]



Stroke

 

600 mm

[image: image11.png]



Pressing-in Speed

1.7 - 18.1 m/min

Drawing-out Speed

1.3 - 14.3 m/min

[image: image12.png]



Operation

Radio Control

[image: image13.png]



Movement

Self-moving

[image: image14.png]



Machine Length

2,370 mm 

Machine Width

1,527 mm 

Machine Height

1,880 mm 

[image: image15.png]



Max. Machine Height

1,375 mm 

[image: image16.png]



Weight

4,200 kg 

[image: image17.png]



The hydraulic pressure setting is adjusted to the international standard (kN). 

[image: image18.png]



Pile Pitching Crane

[image: image19.png]



Max. Lifting Capacity

700 kg

Max. Working Radius

 

3.5 m 

[image: image20.png]



Swing Angle

[image: image21.png]360"




Telescopic Boom

3 Sections

[image: image22.png]



Operation

Radio Control

[image: image23.png]



Mountable Position

Right or Left Side

[image: image24.png]



Weight

 

500 kg

[image: image25.png]



Total Machine Weight

4,700 kg

[image: image26.png]



[image: image27.png]



Piling Dimensions

[image: image28.png]



Min. Curving Radius

2,600 mm

Recommendation

3,800 mm

[image: image29.png]



Corner Installation

L1, L2, R1, R2

[image: image30.png]



Minimum Clearance

Pile Overhead

1,550 mm

Side from Pile Axis

 

750 mm

[image: image31.png]



[image: image32.png]



Power Pack (Crawler Unit)
[image: image33.png]



Unit Type

EU60 Crawler Unit

[image: image34.png]



Power Source

Diesel Engine

Rated Output

44 kW (60PS)

[image: image35.png]



Fuel Tank

135 L 

Hydraulic Oil

190 L 

[image: image36.png]



Noise Level at 7m

60 dB(A)

[image: image37.png]



Crawler Type

GT1

[image: image38.png]



Crawler Operation

Remote Control

Power Source

[image: image39.png]2 Pumps x 2 Motors




Moving Speed

1.2 km/h

[image: image40.png]



Length

2,120 mm

Width

1,700 mm

Height

2,210 mm

[image: image41.png]



Weight

3,500 kg 

[image: image42.png]



[image: image43.png]



Reaction Stand
[image: image44.png]



Length

2,690 mm 

Width

2,100 mm 

Height

 

605 mm 

[image: image45.png]



Weight

1,160 kg 

[image: image46.png]




	 

	 
	[image: image47.jpg]



	 
	
	 


시공문의 : 011-343-7693   유진파일건기 김민호

